This program accepts a list of pole and zero locations and

then calculates the amplitude response for a set of

frequencies.  Real and complex poles can be specified.

However, complex poles are always assumed to occur

in conjugate pairs.  Therefor, only one of a complex

conjugate pole or zero entry per pair is made.  The

other will automatically be created.

New and improved pzsim will accept a much broader range

of frequency scaled root locations without resulting

in an internal overflow.  pzsim will also now calculate

phase response and step response.  A major format difference

from the original program is that every line must end with

a space and a semicolon (i.e., " ;").

Input Format

FS <val> [modif] ;		The starting frequency in Hz.

FE <val> [modif] ;		The ending frequency in Hz.

FP <val> [modif] ;		The number of partitions the

frequency range is to be divided

into.

TS <val> [modif] ;		The starting time in seconds.

TE <val> [modif] ;		The ending time in seconds.

TP <val> [modif] ;		The number of points the time range

is to be divided into.

P <val> [modif] ;		The location of a pole on the real axis.

Z <val> [modif] ;		The location of a zero on the real axis.

PC <val_re> <val_im> [modif] ;	The complex location of a pole.

ZC <val_re> <val_im> [modif] ;	The complex location of a zero.

LOG ;				The output will be calculated in dB

(attenuation).  Only applies to

magnitude response.

RADS ;				The starting, ending, and output printed

frequencies are accepted and printed in

radians instead of Hz.

PHASE ;				Phase response is calculated and printed

in degrees.

STEP ;				The step response is calculated.  At

present, this routine will not work with

multiple poles at the same location.

Modifiers

All input data can be modified by a metric modifier

or an exponential power.  The modifiers are n,u,m,K,M,G,

for ten to the -9,-6,-3,+3,+6,+9 powers respectively.

An exponential modifier can be entered as "E +36",

which is ten to the thirtysixth power.

Output

The output will be two columns of numbers: the first being the

frequency in hertz, the second being the amplitude response

in absolute units or attenuation in dB, phase response

in degrees, or the step response.  In addition to the x-y

coordinates, further information is supplied to the

output for use by the "plot" program.

Multiple Inputs

Multiple sets of simulations can be executed and directed

to the same plot by deliminating between input sets

of information with the "#" character.